CEKENAS END OF TERM ONE EXAMS-2025

441/2

HOME SCIENCE (CLOTHING CONSTRUCTION)

FORM FOUR

MARKING SCHEME

NAME……………………………………………ADM NO…………………….CLASS……………….

|  | DESCRIPTION | MAXIMUM SCORE | ACTUAL SCORE | REMARKS |
| --- | --- | --- | --- | --- |
| 1 | PRESENTATION a) Work well pressed (1) and folded ( ½ ) b) Label ( ½ ) firmly sewn (1) without concealing details ( ½ ) and in a single fabric ( ½ ) c) Pins/needles ( ½ ) unnecessary tacking threads ( ½ ) and loose threads ( ½ ) removed d) Made up for one leg (1) | 1 ½ 2 ½ 1 ½ 1 |  |  |
|  | Sub- total | 6 ½ |  |  |
| 2. | CUTTING OUT a) All five pieces cut out (5 x ½ ) b) Smooth cutting of the short front (1) and on straight grain (1) c) Smooth cutting of the short back (1) and on straight grain (1) d) Smooth cutting of the pocket ( ½ x2) and on straight grain ( ½ x2) e) Cutting of the waist band ( ½ ) and on fold ( ½ ) f) Waistband C.F. on straight grain (1) | 2 ½ 2 2 2 1 1 |  |  |
|  | Sub-Total | 10 ½ |  |  |
| 3. | WAIST DART a) Straight stitchery (1) tapering to nothing (1) b) Correct length of the dart 6.5cm to within 2mm (6.3 to 6.7cm) (1) c) Correct width of the dart 0.8cm to within 2mm (0.6 to 1cm) (1) d) Dart pressed towards CB (1) e) Dart flat on RS ( ½ ) and W.S. ( ½ ) f) Dart secured at the point ( ½ ) | 2 1 1 1 1 ½ |  |  |
|  | Sub-Total | 6 ½ |  |  |
| 4. | IN-SEAM POCKET a) Upper pocket attached to short front (1) with straight stitchery (1) and on R.S together (1) away from seam line (1) b) Lower pocket attached to the short back (1) with straight stitchery (1) and R.S together (1) away from seam line (1) c) Notches matching ( ½ x 2) on W.S. d) Correct position of pocket (by impression) (1) and good hang (1) e) Pocket reinforced at upper (1) and lower point (1) pocket mouth open (1) f) Smooth stitchery of curved edges of pocket bag (2) | 4 4 1 2 3 2 |  |  |
|  | Sub-Total | 16 |  |  |
| 5. | OPEN SEAM (ABOVE AND BELOW POCKET) If not open seam, award zero a) Straight stitchery of seam above ( ½ ) and below pocket (1) b) Seam well neatened above pocket ( ½ ×2) and below (1 x 2) c) Evenness of seam above pocket ( ½×2) and below pocket (2) d) Seam allowance dipped above the pocket ( ½ ) pressed open ( ½ ) and flat on W.S. ( ½ ) R.S ( ½ ) e) Seam allowance clipped below the pocket ( ½ ) and seam pressed open ( ½ ) and flat W.S ( ½ ) and R.S. ( ½ ) | 1 ½ 3 3 2 2 |  |  |
|  | Sub-Total | 11 ½ |  |  |
| 6. | DOUBLE STITCHED SEAM (if not double stitched seam award zero) a) First row of stitchery made with straight stitches b) Raw edges well tucked under c) Second row of stitchery made with straight stitches W.S (1) and on R.S. (1) d) Evenness of seam (1) and correct size 1cm to within 2mm (0.4cm to 0.8cm) (1) e) Flatness of seam on R.S. ( ½ ) and W.S. ( ½ ) f) Fell facing back of the short | 1 1 2 2 1 1 |  |  |
|  | Sub-Total | 8 |  |  |
| 7. | KNIFE PLEATS (if not knife pleats, award zero) a) Pleats pressed towards the same direction ( ½ ) b) Pleats made on three layers of the fabric (1×2) c) Correct number of pleats made (two pleats) (1x2) d) Pleats well pressed and flat at the waist band (1x2) e) Correct width of pleats 1.5cm to within 2mm (1.3cm to 1.7cm (1×2) | 1 2 2 2 2 |  |  |
|  | Sub-Total | 9 |  |  |
| 8. | ATTACHING WAISTBAND a) First attachment line made with straight stitchery (mark by impression 1x2 b) Seam allowance evenly trimmed ( ½ x 2) (see thro light) c) Raw edges well tucked under ( ½ x2) with tacking stitches ( ½ x2) d) Even width of the waistband on R.S. and R.S hemmed side (1 x 2) and correct width 3.5cm to within 2mm (3.3cm to 3.7cm) (1×2) e) Hemming stitches worked from right ( ½ ) to left ( ½ ) f) Hemming stitches even (1) and not showing on R.S. (1) | 2 1 2 4 1 2 |  |  |
|  | Sub-Total | 12 |  |  |
| 9. | HEM MANAGEMENT a) Edge neatened with straight stitchery 1×2 b) Evenness of hem depth (1×2) and raw edges concealed ( ½ x 2) c) Correct depth of finished hem 2.5cm to within 3mm (2.2.cm to 2.8cm) (1 x 2) d) Tacking stitches straight (½ ) with no knots (½ ) e) Fatness of hem on R.S. (1) and W.S. (1) | 2 3 2 1 2 |  |  |
|  | Sub-Total | 10 |  |  |
|  | TOTAL | 90 |  |  |
|  | FINAL MARK = Actual Score 2 | 45 |  |  |