441/2

HOME SCIENCE

PAPER 2

MARKING SCHEME

| 1. | PRESENTATION | MAX. SCORE | ACTUAL SCORE | REMARKS |
| --- | --- | --- | --- | --- |
|  | - Work well pressed and folded - Label clearly written, firmly fixed on a single fabric without concealing details - Unnecessary tacking threads, hanging threads and pins removed - Made up the right half | 2 2 2 1 |  |  |
|  | Sub Total | 7 |  |  |
| 2. | CUTTING OUT AND GRAINING |  |  |  |
|  | - All eight pieces cut out - Smooth cutting of the back bodice and on straight grain at the centre back. - Smooth cutting of the front skirt and on straight grain at the centre front - Smooth cutting of the back skirt and on straight grain at the centre back - Smooth cutting of the back neck facing and on straight grain at the centre back | 4 2 2 2 2 |  |  |
|  | Sub Total | 12 |  |  |
| 3. | SINGLE POINTED DART |  |  |  |
|  | - Straight & firm stitchery - Tapering to nothing - Correct length and width - Well fastened at the tip - Pressed towards the CB | 1 1 1 1 1 |  |  |
|  | Sub Total | 5 |  |  |
| 4 | SHOULDER SEAM (MACHINE FELL SEAM) |  |  |  |
|  | - Straight stitchery of the first line - Straight stitchery of the second line close to the fold - Seam neatly tucked under - Correct width 6mm to within 2mm - Flat on the W.S and R.S - Seam pressed towards the C.B | 1 2 1 1 1 1 |  |  |
|  | Sub Total | 7 |  |  |
| 5 | SIDE SEAM (Open neatened seam if not given; Zero) |  |  |  |
|  | - Straight and firm stitchery - Seam pressed open and flat at arm hole and at the hem - Seam neatened - Correct size of the neatened seam turnings 1cm to within 2mm - Neatened seam turnings even through out the length | 1 1 1 1 1 |  |  |
|  | Sub Total | 5 |  |  |
| 6 | NEATENING OF THE NECKLINE |  |  |  |
|  | - JOINING THE FACINGS - Front and back facing joined using open seam. (If not give zero in this section). - Seam with firm and straight stitchery - Both seam turnings trimmed to 1 cm - Open seam pressed open and flat at the neck line. (b)NEATENING OF THE FREE EDGE - Free edge neatened by edge stitching - Smooth stitchery - Evenness of the edge stitching - Appropriate neatening allowance (0.5cm) (c)NEATENING THE NECKLINE - Neckline joined with smooth stitchery - Seam trimmed and snipped - Well knife edged | 1 1 1 1 1 1 1 1 1 1 1 |  |  |
|  | Total | 11 |  |  |
| 7 | THE JOINING OF THE FRONT BODICE TO FRONT SKIRT AND BACK BODICE TO BACK SKIRT (plain seam used, if not give 0 from (e - f) - The front and back bodice attached to front skirt and back skirt with neatly curved and firm stitchery. - Well reinforced on and off. - Seam well pressed and flat - Seam trimmed to 1cm (to within 2mm) - Two seam turning neatened together with appropriate neatening stitches - Quality of neatening stitches: - Tension - Evenness - Neatened on and off - Single thread used | 2 1 1 1 1 1 1 1 1 |  |  |
|  | Sub total | 10 |  |  |
| 8 | CONTINOUS WRAP OPENING - Correct size of the opening (5cm) - Sleeve binding attached with straight and firm stitchery - Correctly tucked completely concealing the raw edges - Held with hemming stitches - Binding pressed flat | 1 1 1 1 1 |  |  |
|  | Sub total | 5 |  |  |
| 9 | UNDERARM SEAM (French seam if not give zero) - Straight and firm stitchery - Raw edges completely enclosed - Well knife –edged - Even width 0.6cm to within 2mm - Flat seam at the armhole and at the cuff - Seam pressed towards the C.B | 1 1 1 1 1 1 |  |  |
|  | Sub total | 6 |  |  |
| 10 | PREPARATION AND ATTACHMENT OF THE CUFF - Cuff joined with straight and firm stitchery - Seam allowance trimmed and the corner snipped - Well knife edged - Correct size and shape of the cuff - Cuff attached to the sleeve with firm and straight stitchery - Cuff pressed flat | 1 1 1 1 1 1 |  |  |
|  | Sub total | 6 |  |  |
| 11 | PREPARATION AND ATTACHMENT OF THE SLEEVE - Sleeve attached with smoothly curved and straight stitchery - Raw edges and notches matching - Front sleeve joined to the front bodice and vice versa - Seam not trimmed and not neatened - Correct size of the seam allowance - Fullness evenly distributed at the crown - Underarm and side seam meeting at the armhole to within 2mm - Good hang of the sleeve | 1 1 1 1 1 1 1 1 |  |  |
|  | Sub total | 8 |  |  |
| 12 | BUTTONHOLE STITCHES (if not give zero) - Good tension and evenness of the stitches - Fastened on and off - Single thread used - Both ends worked with square edge | 1 1 1 1 |  |  |
|  | Sub total | 4 |  |  |
| 13 | HEM MANAGEMENT - Raw edges completely enclosed by double fold on the garment - Hem managed appropriately - Hem held using even tacking (if not give zero from(c-f) - Good tension of the and evenness of the stitches - Fastened off and on - Hem flat and even in width | 1 1 1 1 1 1 |  |  |
|  | Sub total | 6 |  |  |
|  | G R A N D T O T A L | 90/2 | X/2 |  |