441/2

HOME SCIENCE

(Clothing Construction)

PAPER 2

PRACTICAL

MARKING SCHEME

CANDIDATES NAME__________________________________________    	INDEX NUMBER_____________

NAME OF EXAMINATION TEACHER___________________________	DATE________________________

|  | MAX SCORE | ACTUAL SCORE | REMARKS |
| --- | --- | --- | --- |
| 1. PRESENTATION - Work well pressed (1) and well folded (1) - Label firmly fixed (1) on a single fabric (1) - Pins (1) and unnecessary temporary stitches removed (1) - Made up for the left half (2) | 2 2 2 2 |  |  |
|  | 8 |  |  |
| 2. CUTTING OUT - All seven pieces well cut out (½ x7) - Smooth cutting (1) on straight grain of CB (1) within 2mm - Smoothly cutting (1) on straight grain of CF (1) to within 2mm - Smoothly cut looser edges (1) - Bound opening strip cut on bias (2) | 3½ 2 2 1 2 |  |  |
|  | 10 |  |  |
| 3. POCKET - Hem appropriately finished(1) (i.e. raw edges well neatened(1) or hem firmly fixed) - Even width of hem/ folded part(1) - Pocket sides well tucked under(1) - Pocket fixed with straight stitching (1) close to edges (1) - Pocket stitching pivoted at corners (2) - Pocket mouth well reinforced (2) - Flatness of the pocket R.S (1) and W.S (1) - Size (1), shape (1) of pocket to within 3mm. | 2 1 1 2 2 2 2 2 |  |  |
|  | 14 |  |  |

| 4. SHOULDER DART - Straight stitching ( ½ ) tapering to point (½) - Thread ends well fastened at the point (1) - Correct length of dart(i.e. 5.5cm to within 3mm) (1) - Correct width of the dart (i.e. 6mm) to within 2mm(1) - Dart pressed towards CB(1) - Flatness of the dart R.S (1)and W.S(1) | 1 1 1 1 1 2 |  |  |
| --- | --- | --- | --- |
|  | 7 |  |  |
| 5. SIDE SEAM (open seam, if not give zero) - Joined with straight stitching (1) - Well neatened (1 ×12) - Flatness of seam at armhole R.S ( ½ ) and W.S ( ½ ) - Evenness ( ½ ) of neatened seam allowance and size ( ½ ) | 1 1 1 1 |  |  |
|  | 4 |  |  |
| 6. SHOULDER SEAM (open seam if not give zero) - Seam joined with straight stitching (2) - Seam well neatened (2) - Evenness (1) of neatened seam and size (1) to within 2mm - Flatness of seam at armhole W.S (1) and R.S (1) | 2 2 2 2 |  |  |
|  | 8 |  |  |
| 7. SLEEVE - Good hung (1) - Gathers well distributed(2) at sleeve crown(1) - Dress and sleeve notches matching at front(1) and at back(1) with smooth stitching round armhole(1) If not French seam, give zero from D to J. - Underarm seam (French ) well-trimmed(i.e. no threads on R.S) (1) - Underarm seam made with straight stitching(final stitch) (1) - Seam well knife edged (2) - Seam pressed towards back at cuff(½) and armhole W.S( ½ ) and R.S (1 ) - Flatness of seam at cuff and armhole W.S and R.S (2) - Side seam and underarm seam meeting (1) | 1 3 3 1 1 2 2 2 1 |  |  |
|  | 16 |  |  |
| 8. SLEEVE CUFF: - Cuff interfaced ( ½ ) - Well distributed gathers (1) if pleats ( ½ ) - Quality of cuff attachment by impression of the first line of stitching. (1 ) - Quality of neatening of the cuff on W.S i.e. raw edges evenly tucked under (1) and well stitched (1) - Cuff ends well stitched( ½ x2) and knife edged (1) - Quality of top stitching of cuff - Flatness of cuff R.S (1) W.S (1) - Evenness of cuff width on R.S only | ½ 1½ 1 2 2 1 2 1 |  |  |
|  | 11 |  |  |
| 9. SLEEVE OPENING - Quality of the attaching stitching(by impression) - Quality of neatening binding on W.S i.e. if raw edges are well tucked under( ½ ) and with good stitching( ½ ) - Evenness of binding on R.S(by impression) (2) - Opening base reinforced (2) - Binding ends concealed in the cuffs edges flush to within 3mm(1x2) - Flatness of the opening when lapped(closed) (1) - Equal sides of the opening (1x2) | 2 1 2 2 2 1 2 |  |  |
|  | 12 |  |  |
| Total mark | 90 |  |  |
| Final mark Total mark 2 | X 45 |  |  |