HOME SCIENCE

PAPER 2

(CLOTHING AND TEXTILES)

MARKING SCHEME

|  | PRESENTATION | TOTAL MARKS | ACTUAL SCORE | REMARKS |
| --- | --- | --- | --- | --- |
|  | - Work well pressed (½) and well folded (½) - Label firmly sewn on a single fabric - Unnecessary temporary stitches, threads and pins removed - Made up left half | 1 1 ½ 1 ½ 1 |  |  |
|  |  | 5 |  |  |
|  | Cutting out and graining - All the seven pieces cut out (Back, front, sleeve, cuff, cuff facing, opening strip and pocket) ½x7 - CB smoothing cut (1) and straight grain to within 2mm(1) - CF smoothing cut (1) and straight grain to within 2mm (1) - Smoothing lower edge cut, - Bound opening strip cut on bias (1) | 3 ½ 2 2 1 |  |  |
|  |  | 10 ½ |  |  |
|  | Pocket - Hem appropriately finished (raw edge well neatened) - Even width of the folded part 1 - Pocket sides well tacked under 1 - Pocket fixed with straight stitchery - Pocket stitchery pivoted at the corners 12×3 - Pocket mouth well reinforce 12×3 - Flatness of the pocket RS 12 and WS 12 - Size (1) and shape (1) of the pocket - Superimpose the pattern pocket to within 3mm | 1 1 1 1 112 112 1 2 1 |  |  |
|  |  | 11 |  |  |
|  | Shoulder dart - Straight stitchery (1) tapering to nothing 12 - Threads ends well fastened at the tip - Correct length of the dart 512cm to within 3mm measuring from first stitching line - Correct width of the dart 6mm to within 2mm measured from first stitching line at shoulder seam. - Dart pressed towards CB and flat at the right side. - Flatness of the dart RS12 and WS12 | 112 1 1 1 1 1 |  |  |
|  |  | 612 |  |  |
|  | Side seam (open seam) if not award zero - Joined with straight stitchery - Quality of neatening (1x2) - Flatness of the seam at the armhole RS 2 - Evenness of the neatened seam allowance | 1 2 2 2 |  |  |
|  |  | 7 |  |  |
|  | Shoulder seam (plain seam) - Seam joined with straight stitching line - Well neatened raw edges 12×2 - Evenness of the neatened seam 1×2 - Flatness of the seam at the armhole RS12 and WS12 | 1 1 2 1 |  |  |
|  |  | 5 |  |  |
|  | Sleeve - Good hang - Well distributed gathers at the sleeve head - Dress and sleeve notches matching at front 12 and at the back 12 - Underarm seam (French) well – trimmed (i.e. no threads on RS) - Underarm seam made with straight stitchery - First stitching line made straight (1) mark by impression and seam well knife edged (1) - Evenness at the seam (1) width and appreciate size (1) of the width (0.6 – 1cm) - Seam pressed towards back at cuff 12 and arm hole12 - Flatness of the seam at cuff12 and armhole 12 WS12 and RS12 - Side seam and underarm seam meeting within 2mm | 1 2 1 1 2 2 2 1 2 1 |  |  |
|  |  | 15 |  |  |
|  | Sleeve cuff - Cuff interface - Well distributed gathers - Quality of cuff attachment 12mark by impression the first stitching line - Quality of the neatening the cuff on the WS (raw edges well tucked under (1) and well stitched down (1) - Cuff ends well stitched (1x2) and knife edged (1x2) - Quality of the top stitching the cuff - Flatness of the cuff - Evenness of the cuff width | 1 1 1 2 4 2 1 2 |  |  |
|  |  | 14 |  |  |
|  | Sleeve opening - Quality of the attaching stitchery (mark by impression) - Quality of the neatening of the binding on the WS, that is raw edges neatly tucked under (1) and the quality of the stitches (1) - Evenness of the binding width - No gathers/ folds at the opening base. - Opening base reinforced - Binding ends concealed in the cuff. 12×2 - Binding ends appropriately organized at the cuff – overlap folded (1) to WS and the under lap left unfolded (1) - Binding edges and cuff edges flush (evenly aligned to within 2mm 12×2 - Flatness of the opening when lapped/closed. | 1 2 2 2 2 1 2 2 2 2 |  |  |
|  |  | 16 |  |  |
|  | TOTAL MARKS | 902 |  |  |